Pembrokeshire County Council

Cyngor Sir Penfro

CONDITIONS FOR HOLIDAY SITE FOR STATIC CARAVANS

GENERAL

- The site identification plan attached hereto and numbered XXX is to be considered a part of this licence and permission is granted only in respect of the land shown on the plan edged RED; such land consisting of:

NAME OF SITE:

OS Number:

The total number of caravans that may be stationed on the site at any one time during the period of this licence shall not exceed – XXXX

- No caravan shall be occupied by a greater number of persons than that for which it was originally designed.

- Caravans shall not be occupied other than during the period –

- No tent which does not form part of a family caravan unit shall be placed on site.

- The site shall be maintained in a tidy condition and kept clear of all litter and refuse.

- Such records [including the name and address of the main occupier of each caravan] as may be necessary to give reasonably information as to the observance of the conditions, shall be kept at all times and shall be made available for inspection by an authorised officer of the Council at any reasonable time.

SITE NOTICES

- A sign indicating the name of the site shall be displayed at the site entrance.

- A copy of the site licence with its conditions shall be displayed on site in a prominent position, together with a copy of the current site layout plan.

- Notices shall be displayed on the site, setting out the action to be taken in the event of an emergency.  They shall show where the police, fire brigade, ambulance and local doctors can be contacted, and the location of the nearest public telephone.  The notices shall also give the name and location/telephone number of the site licence holder or his/her accredited representative.  At sites subject to flood risk, warning notices must be displayed giving advice about the operation of the flood warning system

- Where there is overhead electricity lines on the site warning notices shall be displayed on the supports of all these lines and at the site entrance.  Where appropriate these shall warn against the danger of contact between the lines and masts of dinghies and yachts.

- All notices shall be maintained in a clear and legible condition.

- All notices shall be suitably protected from the weather and displayed where possible out of the direct rays of the sun, preferably in areas lit by artificial lighting.  All notices shall be maintained in a clear and legible condition.

SITE LAYOUT

- The licence holder shall provide the Director of Development with a current site layout plan to a scale of not less than 1/500.  The site layout plan shall identify:

- boundaries of the site,

- position of caravan standings,

- roads and footpaths,

- toilet blocks, stores and other buildings,

- foul and surface water drainage,

- water supply

- recreation spaces,

- fire points,

- parking spaces,

- proposed improvements.

- The boundaries of the site shall be clearly marked, for example by fences or hedges.  A 3m wide area shall be kept clear within the inside of all boundaries

- Roads and footpaths shall be designed to provide adequate access for fire appliances.  (Detailed guidance on turning circles etc is available from fire authorities).  Roads of suitable material shall be provided so that no caravan standing is more than 50m from a road.  Where the approach to the caravan is across ground that may become difficult or dangerous to negotiate in wet weather, each standing shall be connected to a carriageway by a footpath with a hard surface.  Roads shall not be less than 3.7m wide, or if they form part of a clearly marked one way traffic system, 3m wide.  Gateways shall be a minimum of 3.1m wide and have a minimum height clearance of 3.7m.  Footpaths shall not be less than 0.75m wide.  Roads shall have no overhead cable less than 4.5m above the ground.  Roads and footpaths shall be suitably lit. Emergency vehicle routes within the site shall be kept clear of obstruction at all times.

- Every caravan shall stand on a hard-standing of suitable material, which shall extend over the whole area occupied by the caravan placed upon it, and shall project a sufficient distance outwards from the entrance of the caravan to enable occupants to enter and leave safely.

- Subject to the following variations, the minimum spacing distance between caravans made of aluminium or other material with similar fire performance properties shall be not less than 5m between units, 3.5m at the corners.  (See specimen layout at Appendix 1).  For those with a plywood or similar skin it shall be not less than 6m.  Where there is a mixture of holiday caravans of aluminium and plywood, the separation distance shall be 6m; and where there is a mixture of permanent residential homes and holiday caravans, the separation distance shall again be 6m.  The point of measurement for porches, awnings etc is the exterior cladding of the caravan.

-	Porches may protrude 1m into the 5m and shall be of the open type.

-	Where awnings are used, the distance between any part of the awning and an adjoining caravan shall not be less than 3m.  They shall not be of the type, which incorporates sleeping accommodation, and they shall not face each other or touch.

-	Eaves, drainpipes and bay windows may extend into the 5m space provided the total distance between the extremities of 2 adjacent units is not less than 4.5m.

-	Where there are ramps for the disabled, verandas and stairs extending from the unit, there shall be 3.5m clear space between them (4.5m if mixture of caravans) and such items shall not face each other in any space.  If they are enclosed, they may need to be considered apart of the unit and, as such, should not intrude into the 5m (or 6m) space.

-	A garage, a shed or a covered storage space shall be permitted between units only if it is of a non-combustible construction (including non-combustible roof) and sufficient space is maintained around each unit so as not to prejudice means of escape in case of fire.  Windows in such structures should not face towards the units on either side.  Car ports and covered walkways should in no circumstances be allowed within the 5 or 6m space.

- One car only may be parked between adjoining caravans provided that the door to the caravan is not obstructed.  Suitably surfaced parking spaces shall be provided where necessary to meet the additional requirements of the occupants and their visitors.  Plastic or wooden boats shall not be parked between units.  Parking spaces shall be provided.

- The gross density shall not exceed 60 caravans to the hectare, calculated on the basis of the usable area (ie, excluding lakes, roads, communal services and other areas unsuitable for the siting of caravans) rather than the total site area.

- There shall be adequate surface water drainage for carriageways, footpaths and paved areas, and for the site generally.

- Space equivalent to at least one-tenth of the total area shall be allocated for children’s games and/or other recreational purposes.

WATER SUPPLY AND SANITARY FACILITIES

(All mobile chalets have their own facilities. All facilities to be maintained in good working order).

- Where caravans have their own sanitary facilities, they must have their own water supply and connection to a facility for the disposal of foul drainage.

- Where caravans do not have their own facilities, communal facilities must be provided in accordance with the Departments sanitary accommodation guidelines

- Water supplies shall be in accordance with the appropriate current water bye-laws and      statutory quality standards.

- A constant supply of piped water shall be made available at water supply points; each consisting of a standpipe and tap situated over a trapped gully, and positioned so that no caravan is more than 90m away

- Waste water disposal points shall be provided so that each pitch is no further than 90m away.

- Particular consideration shall be given to the needs of the disabled in the provision made for water points, toilets, washing points and showers.

- Satisfactory provision shall be made for the disposal of foul drainage, either by connection to a public sewer or sewage treatment works or discharge to a properly        constructed septic tank or cesspool approved by Pembrokeshire County Council.

# REFUSE DISPOSAL

- Every caravan standing shall have an adequate number of suitable non-combustible refuse bins with close-fitting lids.  Arrangements shall be made for the bins to be emptied regularly.  Where communal refuse bins are also provided these shall be of similar construction and housed within a properly constructed bin store.  All refuse must be removed from the site at least once a week and disposed of in a manner approved by the Director of Development

ELECTRICAL INSTALLATIONS

- Sites shall be provided with an electricity supply sufficient in all respects to meet all reasonable demands of the caravans situated on them.

- Such electrical installations, other than Electricity Board works and circuits subject to regulations made by the Secretary of State under Section 16 of the Energy Act 1983 and Section 64 of the Electricity Act 1947, shall be installed, tested and maintained in accordance with the provisions of the Institution of Electrical Engineers’ (IEE) Regulations/British Standard for Electrical Installations for the time being in force, and where appropriate, to the standard which would be acceptable for the purposes of the Electricity (Overhead Lines) Regulations 1988, Statutory Instruments 1988 No. 1057.

- Work (including inspection and testing) on electrical installations and appliances shall be carried out only by competent persons such as the manufacturer’s appointed agent, the electricity supplier, a professionally qualified electrical engineer, a member of the Electrical Contractor’s Association, a contractor approved by the National Inspection Council for Electrical Installation Contracting, or a qualified person acting on behalf of one of the above.  The installations shall be inspected periodically: under IEE Wiring Regulations/British Standard every year or such longer period (not exceeding 3 years) as is considered appropriate in each case.  When an installation is inspected, it shall be judged against the current regulations.

- The inspector shall, within 1 month of such an inspection, issue an inspection certificate in the form prescribed in the IEE Wiring Regulations/British Standard which shall be retained by the site operator and displayed, supplemented or replaced by subsequent certificates, with the site licence.  The cost of the inspection report shall be met by the site operator or licence holder.

- If an inspection reveals that an installation no longer complies with the regulations extant at the time it was first installed, any deficiencies shall be rectified.  Any major alterations and extensions to an installation and all parts of the existing installation affected by them shall comply with the latest version of the IEE Wiring Regulations.

STORAGE OF LIQUIFIED PETROLEUM GAS (LPG)

- LPG storage shall comply with codes of practice from the LP Gas Association;

- Where gas is stored in fixed bulk tanks – code of practice 1 ‘ Bulk storage at fixed installations’.

- Where gas is stored in cylinders – code of practice 7 ‘storage of full and empty LPG cylinders and cartridges’.